Solar Energy of Building Envelopes (SEBE) workflow

SEBE challenge:  Current models of QGIS have a problem running SEBE. This is due to a NumPy version compatibility problem when calculating sun positions. The fix required a software patch:  a “squeeze” to be applied to one element arrays. This fixed the problem and we got SEBE running on one computer.

- UMEP’s pre-processing conversion tool:

  - The Ellerslie Rd average weather file EPW file was converted to a text file using.

  - Our two clipped East and West datasets were used to generate wall height and aspect rasters, excluding under heights under 3 metres.

- SEBE was run on the above layers, to estimate solar radiation for an average year.

- To calculate the actual roof area of each building footprint (compensating for the distortion of area by its slope factor), the following formula was applied to the newly generated average yearly solar radiation rasters:

AA = AP / cos(Si)

AP  = Being the area squared of the raster pixels (in our case 0.52 = 0.25)

Si   = Being the slope of each raster pixel

For example:

( "EnergyyearroofE@1" * 0.25 ) / cos ( ( ( "East_Wall_slope@1" < 65 ) * "East_Wall_slope@1" ) * 3.141 / 180)

- Zonal statistics were then used to calculate the  sum of solar potential for each building roof.

- The resulting outputs were in to five separate classes using Natural Breaks (Jenks) and Inferno colour ramp.

For comparison, the yearly sunlight outputs generated using Python script, were also processed for adjusted roof area and zonal statistics.